CAM-IMX585-M MIPI Camera Module

Product Overview

The CAM-IMX585-M is a high-performance monochrome CMOS image sensor designed for demanding imaging applications in embedded systems. Featuring an advanced Starvis 2 back-illuminated pixel structure, this 8.3MP sensor delivers exceptional low-light performance, high dynamic range, and precise image quality across diverse lighting conditions.

With native 3840×2160 (4K UHD) resolution and support for 10/12/16-bit RAW output, the CAM-IMX585-M enables professional-grade imaging for surveillance, industrial inspection, machine vision, and embedded vision applications. The sensor's MIPI CSI-2 4-lane interface ensures reliable high-speed data transmission to host processors like Raspberry Pi 5 and NVIDIA Jetson.

Note: While the sensor supports 10/12/16-bit RAW output, the current driver configuration operates in 12-bit RAW (R12_CSI2P) mode on Raspberry Pi 5.

The sensor's innovative pixel technology minimizes noise while maximizing light sensitivity, making it ideal for both daylight and challenging low-light environments. Excellent low-light performance ensures clear images even in near-infrared conditions.

Industry Applications

Security & Surveillance (24/7 monitoring)
Industrial & Machine Vision (AOI, robotics)
Broadcast & ProAV (live streaming, drones)
Scientific & Medical (microscopy, diagnostics)
Automotive (ADAS, surround-view cameras)

Key Features

• 8.3MP 4K resolution with Starvis 2 technology
• Monochrome sensor for enhanced low-light sensitivity
• MIPI CSI-2 4-lane high-speed interface
• 10/12/16-bit RAW output capability (currently 12-bit)
• Excellent low-light performance with back-illuminated pixels
• Compatible with Raspberry Pi

Sensor Specifications

| Parameter | Specification |
| --- | --- |
| Product Model | CAM-IMX585-M |
| Sensor Model | Sony IMX585 (Monochrome) |
| Sensor Type | CMOS (Mono) |
| Pixel Technology | Starvis 2 |
| Effective Resolution | 8.3MP |
| Optical Format | 1/1.2" |
| Pixel Size | 2.9μm × 2.9μm |
| Active Pixels | 3840 (H) × 2160 (V) |
| Max Resolution | 3840×2160 (4K UHD) |
| Supported RAW Output | 10/12/16-bit |

| Current Working Mode | 12-bit RAW (R12_CSI2P) |
| --- | --- |
| Frame Rate | 30fps @ 4K |
| Dynamic Range | High WDR Support |
| Low Light Performance | Excellent low-light performance |
| Power Consumption | Low power design |

Video Format & Resolution/Frame Rate

Detected Camera Modes (Actual - Raspberry Pi 5)

| Resolution | Frame Rate | Pixel Format | Notes |
| --- | --- | --- | --- |
| 1928×1090 | 30.00 fps | R12_CSI2P | Cropped from 4K |
| 3856×2180 | 30.00 fps | R12_CSI2P | Native sensor readout |

Note: Current driver operates in 12-bit RAW mode. 10-bit and 16-bit modes available with driver configuration changes.

Detection Command: rpicam-hello --list-cameras

Hardware Specifications

Lens Specifications

| Parameter | Specification |
| --- | --- |
| Lens Mount | CS-mount / M12 |
| Focal Length | Variable |
| Field of View | Depends on lens |

Interface Specifications

| Parameter | Specification |
| --- | --- |
| Interface Type | MIPI CSI-2 |
| Data Lanes | 4-lane |
| Data Rate | 1.5 Gbps per lane |
| Connector | 22-pin FPC |
| Compatibility | Raspberry Pi |

Physical Specifications

| Parameter | Specification |
| --- | --- |
| Dimensions | 38mm × 38mm |